SoniX – Battery Calculations

Current Consumption and Estimated Battery Runtime

# 1. Current Consumption by Each Component

| No. | Component | Current (mA) |
| --- | --- | --- |
| 1 | MPU6050 | 1 |
| 2 | MLX90614 (IR non-contact temperature sensor) | 2 |
| 3 | VEML6075 UV sensor | 0.48 |
| 4 | MQ-7 gas sensor | 150 |
| 5 | Buzzer | 10 |
| 6 | ESP32-C3 Mini (Wi-Fi ON) | 150 |

# 2. Total Current Consumption

Total current = 1 + 2 + 0.48 + 150 + 10 + 150 = 313.48 mA

# 3. Battery Specification

Battery capacity: 2500 mAh

Nominal battery voltage: 3.7 V (Li-ion)

Maximum Charged Voltage:4.2V(Li-ion)

# 4. Estimated Battery Runtime

Ideal runtime = Battery capacity (mAh) ÷ Total current (mA)

Ideal runtime = 2500 ÷ 313.48 = 7.97 hours

Note: This is an ideal estimate based on the current values supplied in the handwritten calculation. Actual runtime may be lower because of ESP32 Wi-Fi peaks, sensor warm-up/current variation, voltage-conversion losses, battery condition, and alert activity. The MQ-7 module may draw substantial heater current; verify the exact module's current and supply requirements before finalizing the battery design.

# 5. Calculation Summary

| Total listed current | 313.48 mA |
| --- | --- |
| Battery capacity | 2500 mAh |
| Ideal calculated runtime | 7.97 hours |
| Practical runtime | To be confirmed by prototype testing |